# COM T3, I2C_SPI assignment

## Part 1

The slave address for the temperature sensor is 0x94. To use this address in the wire library, the value needs to be shifted to the right once. This results in a value of 0x4a. To request data from the sensor, there needs to be written to the correct addresses. The address for the low byte is 0x02 and for the high byte it’s 0x00. These register addresses are found in (Datasheet STTS751, 2019). The low byte returns a binary value that needs to be converted into the correct value. To do so I took the maximum binary value and the maximum temperature value for it and divided this by each other. This resulted in 240 / 0.9375 = 256. This calculated value is used to divide the low byte with.

## Part 2 (Optional)

For the second part I decided to go with the relative humidity of the HTS221 sensor. I used the same approach I’ve done in part 1. Therefore, I first looked for the correct addresses (slave address and registers). The slave address is, as mentioned on the schematic, 0xBE, and the register addresses are 0x28 for the low byte and 0x29 for the high byte. As mentioned in (Datasheet HTS221, 2016) the humidity is calculated based on the data from the two registers. First, from each register the data is divided by 2. Secondly, the two outcomes are added together and lastly is divided with 2 again.

## Part 3

The Sparkfun Qwiic Joystick can be connected to the I2C lines of the RedBoard. By accessing the horizontal and vertical registers the values of the Joystick can be read. Both of the horizontal and vertical registers have to addresses each. By only reading the first register, the Joystick value range from 0 to 256. When making use of the high byte, the Joystick value ranges from 0 to 1023. Since it is not necessary to read both, the low and high byte, I will only be using the low register. The register address are 0x03 and 0x05, and the slave address is 0x20. These register values are found in (Qwiic Joystick). To use the retrieved values, they have been remapped to use them more easy. Based on the values of the Joystick the values from the assigned sensor will be displayed in the serial monitor. The assigned sensor are the ones discussed in the previous parts.

## Part 6

Regarding to the SPI master, it transfers data to the SPI slaves based on its serial input. It will only transfer data when there is an ‘a’ or ‘b’ received. To transfer the data to the slave the slave pin is turned low, data is transferred and then the slave pin is set high again. For the SPI slave to receive this data the device needs to be put into the slave mode and master in. When data is being received, the data from the SPI data register will be transferred to my own global variable and based on this, the LED will be turned on or off.

# References

Datasheet HTS221. (2016, August). From st.com: https://www.st.com/resource/en/datasheet/hts221.pdf

Datasheet STTS751. (2019, May). From st.com: https://www.st.com/resource/en/datasheet/stts751.pdf

Gammon, N. (2015, May). SPI - Serial Peripheral Interface - for Arduino. From gammon.com: http://www.gammon.com.au/spi

Qwiic Joystick. (n.d.). From Sparkfun.com: https://learn.sparkfun.com/tutorials/qwiic-joystick-hookup-guide?_ga=2.71618320.286045724.1599660497-229224066.1599660497