# Lab_02 : System Timer

## Part A

In the first part of the assignment , there had to be a LED toggled with a SysTick interval of 1 second. To make use of the SysTick function, the reload value had to be calculated. I decided to go with a SysTick interval of 1ms. The reload value is calculated in Figure 1 : Calculation RELOAD.

Reload=SysTick interval ∙Clock Frequency

Reload=1 ×10-3 ∙16 × 106-1

Reload=16 000-1

Figure 1 : Calculation RELOAD.

Regarding to creating the actual delay, I made use of reading the count flag bit. When the clock cycle is completed, the bit changes into a 1. There for when creating a delay based on this, each tick is 1ms.

## Part B

For the second part of the assignment, the SysTick handler had to be implemented as well. I approached the assignment the same way as I had done with the previous part of the assignment. I started of with calculating the SysTick interval. Since working with a SysTick interval of 1ms worked very well during the previous part of the assignment, I decided to go with it again for this part of the assignment. Therefore, the calculation can be found in Figure 1 : Calculation RELOAD.

## Part C

For the last part of the assignment, I made use of my code from Part B. My idea was to make the SOS single in morse code by toggling the LED. When trying to make this work, I ran into another problem; I could toggle the bit to make the LED blink, but when making use of the simple |= and &= to toggle the LED, it did not want to work. Therefor I was unable to make the idea come to live the way I wished for. Since only the toggling of the bit worked I was unable to make the delay in between the dashes as short as in the delay between the dots.

# References

Lab_02_SysTick. (n.d.). From Git.Fhict: https://git.fhict.nl/technology/t-sem3-cb/-/blob/master/es/Timers/Lab_02_SysTick.pdf

timers_systick. (n.d.). From git.fhict: https://git.fhict.nl/technology/t-sem3-cb/-/blob/master/es/Timers/timers_systick.pptx