Technical Document

Messaging Structure

Edorado Marine

Dordrecht, The Netherlands

Date : 09.06.2923

Version : 2.0

Status : Final Version

Author : Casey Stijlaart

Table of Contents

1. 	Introduction	3

2. 	The Message Structure	3

3. 	The Command API	4

4.	Response	4

5. 	Examples	5

Table of Tables

Table 1. The Message Format	3

Table 2. The Command API	4

Table 3. The GetPosition Request Message	5

Table 4. The GetPosition Success Response Message	5

Table 5. The GetPosition Error Response Message	6

# 1. 	Introduction

Within the system, messages will be sent via the communication protocol. To ensure a proper communication rules and standards need to be set. Therefore, to create a proper messaging protocol, there has to be looked at trough what type of communication it is sent. Based upon a discussion, with Kai Harten and Peter Meijer, during the Sprint 0 demo, it has been determined to start with Serial Communication. Within a send data packet, there is made use of multiple bytes. Each Byte with its specification on what kind of data it will carry.

# 2. 	The Message Structure

Each message follows the same rules regarding its format. The message format consists of the following components:

| Field | Size (bytes) | Endianness | Description |
| --- | --- | --- | --- |
| Start Bytes | 4 | N/A | Unique identifier for the start of the message (ASCII characters 'E', 'D', 'O', '8') |
| Payload Size | 2 | Big-endian | Size of the payload, represented as an unsigned integer |
| Command | X | N/A | Variable-length payload, where X is the value indicated by the payload size field |
| CRC | 2 | Big-endian | Cyclic Redundancy Check the value of the message |

Table 1. The Message Format

The format of the message can be described as follows:

- Start Bytes: The message starts with 4 bytes that form the “start bytes” of the message. In this case, the start bytes are the ASCII characters ‘E’, ‘D’, ‘O’, and ‘8’. These bytes serve as a unique identifier for the start of the message and allow the receiver to detect the beginning of a new message.

- Payload Size: Following the start bytes, 2 bytes indicate the size of the payload. The size is represented as an unsigned integer and is encoded in big-endian format. This means that the most significant byte appears first in the message.

- Payload: After the payload size field, the message contains a variable-length payload of X amount of bytes, where X is the value indicated by the payload size field. The content of the payload is application-specific and can include any data or information that needs to be communicated between the sender and receiver.

- CRC: Finally, the message ends with a 2-byte CRC (Cyclic Redundancy Check) value that provides a checksum for the message. The CRC is used to detect any errors that may have occurred during transmission and can help ensure message integrity. Like the payload size field, the CRC value is encoded in big-endian format.
By adhering to this format, both the sender and receiver can reliably communicate with each other and ensure the integrity of the messages being exchanged.

# 3. 	The Command API

Different commands can be sent and requested. The defined commands are as follows:

- GetDeviceId

- GetDeviceStatus

- GetConnectedActuators

- SetPositionAbsolute

- GetPosition

| Id | Name | Parameters | Return Values | Description |
| --- | --- | --- | --- | --- |
| 0x00 | GetDeviceId | N/A | 128-bit Device ID | The universally unique device ID |
| 0x01 | GetDeviceStatus | N/A | 32-bit Status structure | Request the current device status |
| 0x02 | GetConnectedActuators | N/A | An 8-bit integer from 0 to 4 | Get the number of connected actuators to the module |
| 0x03 | SetPositionAbsolute | An 8-bit integer from 0 to 100 | 8-bit command status | Set the absolution position in percentage |
| 0x05 | GetPosition | N/A | An 8-bit integer from 0 to 100 | The current actuators’ position in percentage |
| 0x06 | GetSinglePosition | 8-bit actuator number | 8-bit The actuator position in percentage |  |
| 0xEE | Error | 8-bit error type | N/A | The error response |

Table 2. The Command API

# 4.	Response

There are two possible responses that can be given upon request. These are the success and the error return. The response of a device with success is the command id ^ 0x80 and the additional return values. And The response of a command with failure is 0xEEEE + byte with the type of error

# 5. 	Examples

In the given example the GetPosition is requested and the two possible responses.

| Index | Value | Description |
| --- | --- | --- |
| 0 | 'E' | First start character -> start[0] |
| 1 | 'D' | Second start character -> start[1] |
| 2 | 'O' | Third start character -> start[2] |
| 3 | '8' | Fourth start character -> start[3] |
| 4 | 0x00 | The first byte of payload size -> payload_size[0] |
| 5 | 0x01 | Second byte of uint16_t payload size -> payload_size[1] |
| 6 | 0x05 | The first byte of payload -> payload[0] -> command.id |
| 7 | 0xEB | First byte of uint16_t CRC -> CRC[0] |
| 8 | 0x61 | Second byte of uint16_t CRC-> CRC[1] |
| Index | Value | Description |

Table 3. The GetPosition Request Message

| Index | Value | Description |
| --- | --- | --- |
| 0 | 'E' | First start character -> start[0] |
| 1 | 'D' | Second start character -> start[1] |
| 2 | 'O' | Third start character -> start[2] |
| 3 | '8' | Fourth start character -> start[3] |
| 4 | 0x00 | The first byte of payload size -> payload_size[0] |
| 5 | 0x02 | Second byte of uint16_t payload size -> payload_size[1] |
| 6 | 0x85 | The first byte of payload -> payload[0] -> command.id |
| 7 | 0x0F | command.data[0] |
| 7 | 0xEB | First byte of uint16_t CRC -> CRC[0] |
| 8 | 0x61 | Second byte of uint16_t CRC-> CRC[1] |

Table 4. The GetPosition Success Response Message

| Index | Value | Description |
| --- | --- | --- |
| 0 | 'E' | First start character -> start[0] |
| 1 | 'D' | Second start character -> start[1] |
| 2 | 'O' | Third start character -> start[2] |
| 3 | '8' | Fourth start character -> start[3] |
| 4 | 0x00 | The first byte of payload size -> payload_size[0] |
| 5 | 0x02 | Second byte of uint16_t payload size -> payload_size[1] |
| 6 | 0xEE | The first byte of payload -> payload[0] -> command.id |
| 7 | 0x0F | command.data[0] |
| 7 | 0xEB | First byte of uint16_t CRC -> CRC[0] |
| 8 | 0x61 | Second byte of uint16_t CRC-> CRC[1] |

Table 5. The GetPosition Error Response Message